YPR-IN-BAB-QSF-720-02-28_CHECKLIST FOR RT

Contents

1.0	CHECKLIST OF RT:…………………………………...……….………..…………………….2

2.0                  Do's & Don'ts guidelines as per M/s AERB-Mumbai…………………………………….3

CHECKLIST FOR RT

| RADIOGRAPHY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | TITLE: WORK PERMIT FOR RADIOGRAPHY |  |  |  |  |
| AREA : |  |  | DATE: |  |  |
| JOB DESCRIPTION : |  |  | TIME: |  |  |
| CONTRACTOR: |  |  |  |  |  |
| RADIOGRAPHY AGENCY: |  |  |  |  |  |
| SL.NO | DESCRIPTION | YES | NO | NOT REQD | REMARK |
| 1 | Strength of source:…………….Curie |  |  |  |  |
| 2 | Whether only authorized and licensed radiographer is appointed for radiography? |  |  |  |  |
| 3 | Whether the radiographer is using dosimeter /Personal Alarm Monitors and TLD Film badge/ ? |  |  |  |  |
| 4 | Whether the barricading with proper warning label is displayed at the location of radiography? |  |  |  |  |
| 5 | Radiography camera operating system is in working condition or not? |  |  |  |  |
| 6 | Whether survey meters are in working condition? |  |  |  |  |
| 7 | Whether the exposure rate at barricading identified by using radiation survey meter? |  |  |  |  |
| 8 | Whether proper communication is made in the area about timing of the activity, type of source and its hazards with precautionary measures? |  |  |  |  |
| 9 | Job is being maintained and supervised by authorized person |  |  |  |  |
| 10 | Whether non working supervisor is present at barricading point? |  |  |  |  |
|  | AUTHORIZING PERSON:. | SUPERVISOR |  |  |  |
|  | YFI NDT Project Co-Ordinator | CONTRACTOR |  | NDT AGENCY |  |
| NAME: |  |  |  |  |  |
| SIGN: |  |  |  |  |  |
| TIME |  |  |  |  |  |

Do's & Don'ts guidelines as per M/s AERB-Mumbai

| Do's & Don'ts guidelines as per M/s AERB-Mumbai |  |  |  |  |
| --- | --- | --- | --- | --- |
| S NO | DO’s |  | DON’Ts |  |
| 1 | Before starting radiography work, radiographer/ site in charge should check that all safety equipment (survey meter etc.) is in working condition and calibration is valid | ✔ | IGRED is moved out of the pit room for radiography job without verifying the interlocks and safety mechanism . | X |
| 2 | Radiographer/site in charge should verify the IGRED for working condition of interlocks and other safety features, if any | ✔ | Use of safety equipment (survey meter etc.) for which calibration is not valid. | X |
| 3 | Radiographer/site in charge should confirm that emergency equipment is available and in working condition. | ✔ | Emergency handling equipment are available at office of the agency but not at site. | X |
| 4 | Set up the cordoning off area and the necessary barriers with intimation to the CAP. | ✔ | Without set up of operation is started. | X |
| 5 | Confirm the radiation level outside the cordoning off area by use of a survey meter | ✔ | CAP not informed about radiography work being carried out. | X |
| 6 | Secure and store the IREDs when not in use. | ✔ | After radiography job radiography device left unattended at the site. | X |
| 7 | Use appropriate collimators. | ✔ | Inappropriate tools are used by the radiographers/ site in charge | X |
| 8 | Confirm that there are no other people working inside the cordon-off area. | ✔ | Radiography work is undertaken in public/non radiation workers occupied area. | X |
| 9 | Use clear working signs and signals and Red warning lamps. | ✔ | Controlled area is accessible to other people | X |
| 10 | Observe the access restrictions, however remote it may seem from the location of the source | ✔ | Radiography device is left unattended | X |
| 11 | DO familiarize factory/plant/CAP workers with all the warning signs and signals the radiographers use. | ✔ | Assisting staff/helper is not familiarize with warning signs and signals | X |
| 12 | Never leave the radiography device unattended. | ✔ | Not reported regarding the safety concerns | X |
| 13 | Shouldn’t allow anyone into the restricted area/cordoning area | ✔ | Radiography device is tampered | X |
| 14 | TLD badges should NOT be left at the radiography site/inside radiography enclosure | ✔ | TLD badges are not stored in a radiation free area | X |
| 15 | TLD badges should NOT be kept in accessories/tool bag, as it may lead to non genuine exposure to the TLDs raising false concern of excessive exposure to personnel | ✔ | TLD badges are kept/forgotten near the radiation source or IGRED | X |